EX NO: 7             Examining Network Address Translation (NAT) using Cisco Packet

DATE:27.08.24                                                     Tracer

Aim:

To Examine Network Address Translation (NAT) using Cisco Packet Tracer

- Setting Up the Network Topology

  - Devices Required:

    - Two PCs (for testing connectivity)

    - Two router (to configure NAT)

    - Two switch (to connect the PCs and the router)

    - One server (to simulate an external network, like the internet)

  - Steps:

1.Place the Devices: Drag and drop the required devices onto the workspace.

2.Connect the Devices: Use the appropriate cables (copper straight-through for PCs to the switch, copper cross-over for switch to router) to connect the devices.

3.Assign IP Addresses:

  - Assign private IP addresses (10.10.10.0/20) to the PC1 and the router1 Internal interface.

  - Assign a public IP address (50.50.50/30) to the router1 external interface.

  - Assign an IP address to the server that simulates an external network (10.10.10.2).

  - Assign private IP addresses (20.20.20.0/20) to the PC2 and the router2 Internal interface.

  - Assign a public IP address (60.60.60/30) to the router2 external interface.

- Configuring NAT on the Router Steps:

1.Access the Router CLI: Click on the router and go to the CLI tab.

2.Enter Global Configuration Mode:

3.Configure Interfaces:

Set up the internal and external interfaces:

Configure NAT Overload (PAT):

Define an access list to match the internal IP range:

  - Configure NAT to translate the internal addresses to the external address.

  - This configures PAT (Port Address Translation), which allows multiple internal IPs to share a single external IP.

- Testing NAT

Steps:

1.Ping from a PC to the External Network:

From one of the PCs, open the command prompt and try to ping the external server (e.g., ping 200.0.0.2).

2.Verify NAT Translations:

On the router CLI, check the NAT translation table to see the active translations:

3.Observe the Output:

The NAT translation table should show the mapping of the internal private IP addresses to the external public IP.

- Observing the Traffic

  - Use the simulation mode in Packet Tracer to visually observe the NAT process as packets move from the internal network to the external network.

- Saving the Configuration

Don't forget to save the configuration on the router to avoid losing the settings:

Result:

Thus,we successfully, we examine  network address Translations(NAT) using Cisco Packet Tracer.